### MAROOCHY SHIRE COUNCIL PLANNING SCHEME

### POLICY NO. DC4

### STORMWATER QUALITY

### DC 4.1	INTRODUCTION

(1)	Conventional urban development generally increases stormwater runoff quantity and decreases stormwater runoff quality.

(2)	The increased stormwater quantity (i.e. flow/volume) is a direct result of converting pervious surfaces to impervious surfaces and installing collection and drainage systems to concentrate and remove stormwater as efficiently as possible.

(3)	The decrease in stormwater quality results from the pollutants that are generated in urban environments combined with the improved mobility that conventional drainage systems offer these pollutants.

(4)	The consequences of urban development on our waterways are felt both locally and regionally through erosion, siltation, pollution, flooding and the loss of sensitive ecosystems and habitats.

(5)	It is for these reasons that Council is committed to improving the health of the Shire’s waterways.

(6)	This Planning Scheme Policy is the mechanism to partially fund, via developer contributions, the construction of the Trunk Stormwater Quality Treatment Network.

(7)	The scope of infrastructure for which funding is obtained via this planning scheme policy is limited to Stormwater Quality Treatment infrastructure that relates to the achievement of Shire wide environmental values.

NOTE DC 4.1.7 INTERNAL STORMWATER QUALITY TREATMENT INFRASTRUCTURE FOR LOCAL ENVIRONMENTAL VALUES

- The ‘internal’ Stormwater Quality Treatment infrastructure (eg. Vegetation filters, Physical and Biological filters, Aquatic Environments) is the responsibility of the Developer and will be applied as a condition in any development approval.

- Where ‘internal’ Stormwater Quality Treatment infrastructure is required it is to be designed and constructed by suitably qualified personnel and provided generally in accordance with Best Management Practices.

- For the purpose of clarity it is recorded that the Council is not responsible for the construction or the cost of any part of internal Stormwater Quality Treatment facilities.

- Infrastructure contributions payable by a Developer pursuant to this Planning Scheme Policy are additional to the ‘internal’ infrastructure that the Developer is required to provide as part of a development.

(8)	The provisions in this planning scheme policy relate to the Infrastructure Contributions for the trunk Stormwater Quality Treatment network as follows -

- The Future Stormwater Quality Treatment infrastructure (see section DC 4.2);

- The desired standard of service for Stormwater Quality Treatment infrastructure (see section DC 4.3);

- The estimated establishment cost of future Stormwater Quality Treatment infrastructure (see section DC 4.4);

- The estimated establishment cost of future Stormwater Quality Treatment infrastructure to be funded by the contribution (see section DC 4.5);

- Infrastructure contributions and calculations (see sections DC 4.6 and Schedule DC 4).

### DC 4.2	FUTURE TRUNK STORMWATER QUALITY TREATMENT INFRASTRUCTURE

(1)	The future Stormwater Quality Treatment infrastructure to be provided across the Shire is shown on Figure 4.2.1 (refer Appendix 1).

DC 4.3	DESIRED STANDARD OF SERVICE FOR STORMWATER QUALITY TREATMENT INFRASTRUCTURE

(1)	The desired standard of service for stormwater quality is outlined in Appendix 2.

### DC 4.4	ESTIMATED COST OF STORMWATER QUALITY TREATMENT INFRASTRUCTURE

(1)	The estimated establishment cost of Stormwater Quality Treatment Infrastructure is outlined in Table DC 4.4.1 —

### TABLE DC 4.4.1   STORMWATER QUALITY TREATMENT TRUNK INFRASTRUCTURE COSTS ($)

| Conceptual Treatment Styles | Structural Measures | Aquatic Environments | Physical and Biological Filters | Total |
| --- | --- | --- | --- | --- |
| Conceptual Treatment Costs | $76,529,629 | $71,353,135 | $102,259,815 | $250,142,580 |

DC 4.5	PROPORTION OF STORMWATER QUALITY TREATMENT TRUNK INFRASTRUCTURE ESTABLISHMENT COSTS TO BE FUNDED BY INFRASTRUCTURE CONTRIBUTIONS

(1)	The proportion of trunk Stormwater Quality Treatment infrastructure costs attributable to infrastructure contributions is outlined in Table DC 4.5.1.

TABLE DC 4.5.1   PROPORTION OF STORMWATER QUALITY TREATMENT TRUNK INFRASTRUCTURE COSTS SUBJECT TO INFRASTRUCTURE CONTRIBUTIONS ($)

| Level Of Works | Costs Not Subject To Infrastructure Contributions | Costs Subject To Infrastructure Contributions |
| --- | --- | --- |
| Conceptual Treatment Costs | $125,795,560 | $124,347,020 |

NOTE DC 4.5 PROPORTION OF STORMWATER QUALITY TREATMENT INFRASTRUCTURE COSTS TO BE FUNDED BY INFRASTRUCTURE CONTRIBUTIONS

- The costs associated with achieving the community’s Shire wide environmental values are to be shared across the full community.

- This Policy requires new development to contribute a proportion of costs towards the achievement of the Shire wide environmental values that relates to the proportion of future development to ultimate development levels.

- Consequently, new development is required to comply with local environmental values and contribute towards Shire wide values.

- The remainder of the costs to achieve Shire wide environmental values will be met by Council’s capital works program.

# DC 4.6	INFRASTRUCTURE CONTRIBUTIONS AND CALCULATIONS

(1)  Those areas of the Shire and the type of development applications subject to Stormwater Quality Treatment infrastructure contributions together with the method of calculating the contribution is outlined in Schedule DC 4.

SCHEDULE DC 4: STORMWATER QUALITY TREATMENT TRUNK INFRASTRUCTURE CONTRIBUTIONS SCHEDULE

AREAS WHERE INFRASTRUCTURE CONTRIBUTIONS APPLY

(1)	All urban areas of the Shire are subject to a Shirewide infrastructure contribution.

(2)	Those urban areas of the Shire subject to the stormwater quality treatment infrastructure contribution are outlined in Table 1 and the boundaries of the Planning Areas and the various precinct classes within each Planning Area are shown on the Planning Area Maps found in Volume 3 of this Planning Scheme.

TABLE 1   PLANNING AREAS SUBJECT TO STORMWATER QUALITY TREATMENT TRUNK INFRASTRUCTURE CONTRIBUTIONS

| PLANNING AREA | SHIREWIDE |
| --- | --- |
| ALEX HEADLAND/COTTON TREE (7) |  |
| BLACKALL RANGE (19) |  |
| BLI BLI (13) |  |
| BUDERIM (6) |  |
| CENTRAL HINTERLAND (27) |  |
| COOLUM BEACH (11) |  |
| EUDLO CREEK VALLEY (21) |  |
| EUMUNDI (17) |  |
| KENILWORTH (18) |  |
| KULUIN/KUNDA PARK (8) |  |
| MAROOCHY RIVER PLAINS (23) |  |
| MAROOCHYDORE (1) |  |
| MARY RIVER VALLEY (30) |  |
| MOOLOOLABA (4) |  |
| MOUNTAIN CREEK (5) |  |
| MOUNTAIN CREEK VALLEY (20) |  |
| MT COOLUM (10) |  |
| NAMBOUR (2) |  |
| NORTH SHORE (9) |  |
| NORTHERN COASTAL PLAINS (25) |  |
| NORTHERN HINTERLAND (26) |  |
| OBI OBI CREEK VALLEY (29) |  |
| PALMWOODS (14) |  |
| PETRIE/PAYNTERS CREEK PLAINS (22) |  |
| SIPPY DOWNS (3) |  |
| SOUTH PEREGIAN (12) |  |
| SOUTHERN HINTERLAND (28) |  |
| WOOMBYE (15) |  |
| YANDINA (16) |  |
| YANDINA CREEK VALLEY (24) |  |

# APPLICATION OF CONTRIBUTION

(3)	Subject to clause (3A), Stormwater quality treatment infrastructure contributions apply to every development application that involves –

(a)	Reconfiguration a lot; or

(b)	A material change of use.

(3A) The following uses are exempt from paying stormwater quality infrastructure contributions –

(a)	all uses defined within the ‘Rural Use’ category as defined in the Planning Scheme;

(b)	uses defined as ‘Extractive Industry’ or ‘Home-based business’ (provided equivalent demand for a detached house is not exceeded);

(c)	a material change of use for a detached house; or

(d)	Non-Complying Self-Assessable Development as defined in Planning Scheme Policy DCA-Administration.

# DETERMINATION OF STORMWATER QUALITY TREATMENT TRUNK INFRASTRUCTURE UNIT RATES

(4)	For the purpose of determining infrastructure contributions towards Stormwater Quality Treatment infrastructure, proportions of the costs to achieve the Shirewide Environmental Values were determined for each catchment based upon percentages of future populations and percentages of developable areas.

(5)	Costs to achieve the Shirewide Environmental Values were averaged across all urban catchments within the Shire.

NOTE 1 SCHEDULE DC4

Note on Apportioning Costs

1) It would be possible to apportion the costs of Stormwater Quality Treatment infrastructure to each of the catchments individually however it is likely that this would bias or burden some catchments whose physical topographies limit the potential to treat stormwater runoff whilst still contributing significant pollutant loads to the receiving waters.

(6)	The stormwater quality treatment infrastructure unit rate for each planning area is 2,192.

(7)	A typical dwelling unit (with an assumed population base of 2.75 persons per dwelling unit) has been adopted as the Demand Unit for determining the infrastructure charges.

NOTE 2 SCHEDULE DC4

Note on Demand Units

- Residential Development

The dwelling unit in all residential precincts is treated as the baseline indicator of demand, generating one demand unit. For example a ten lot subdivision generates ten demand units, whilst in a multi storey residential precinct, a building containing twenty apartments generates 20 demand units.

b.	Multi Storey Residential Development

As per residential development the costs are apportioned on population projections. It is recognised that a multi storey residential development may have a lower impact on stormwater quality per person than a traditional residential development of a similar population. However, achieving the community’s Shire-wide Environmental Values provides a benefit to the entire population of the Shire and hence this strategy seeks to spread the costs across the whole population.

c. 	Commercial or Industrial Development

The cost apportioning to commercial or industrial development is less straightforward than residential development as it is difficult to assume a population for a commercial or industrial site to derive the apportioned cost. Unlike the residential development costs, this Policy suggests that costs apportioned to commercial or industrial development be impact based and relate to the site area. To maintain a link to the principles used to derive costs applied to residential development typical impervious areas of residential and commercial/industrial developments were considered.

The Queensland Urban Drainage Manual (QUDM) suggests percentages of impervious area for residential, commercial/industrial and central business areas of approximately 50%, 90% and 100% respectively.  If residential development is considered as one demand unit and assumed to represent a development area of 1000m2 (including roadway) then a commercial or industrial site will attract apportioned costs at a rate of 1.8 (90/50) demand units per 1000m2 or more simply 0.18 demand units per 100m2 site area and a central business site (being land within the Town Centre Precincts) will attract apportioned costs at a rate of 2 (100/50) demand units per 1000m2 or 0.2 demand units per 100m2 of site area.

# DETERMINATION AND CALCULATION OF STORMWATER QUALITY TREATMENT TRUNK INFRASTRUCTURE CONTRIBUTIONS

(8)	The stormwater quality treatment infrastructure contribution for any proposed development is to be calculated as follows –

[(A – B) – C] x D x E

Where

A (being proposed demand) is –

i.	For reconfiguring a lot the stormwater quality treatment infrastructure demand factor for 	the Land or lots (excluding any Dedicated Lots) included in the development application 	based on the method creating the higher level of demand calculated using the rates 	outlined in Table 2 (a) and Table 2 (b).

ii.	For a material change of use the stormwater quality treatment infrastructure demand 	factor for the use or Land included in the development application based on the method 	creating the higher level of demand calculated using the rates outlined in Table 2(a) and 	Table 2(b).

iii.	For a material change of use where an existing building or existing work is proposed to 		be changed or extended or a new building or work is proposed to be erected on land 			occupied by an existing use the stormwater quality treatment infrastructure demand 			factor for the use included in the development application calculated using the rates 			outlined in Table 2 (b).

B	(being existing use demand entitlements) is –

    - For vacant land, the stormwater quality treatment infrastructure demand factor allowed for a single detached house (1cu) or where previous infrastructure contributions have been paid to Council the demand on which the previous contributions were based.

    - Otherwise, the existing use demand entitlement.

C	is any applicable infrastructure credit for the land (granted as a result of providing advanced funding for the construction of trunk infrastructure or contributing trunk infrastructure) as outlined in the Register of Infrastructure Contributions and Credits.

D	is the stormwater quality treatment infrastructure unit rate as outlined in paragraph (6) of this Schedule.

E	Is the stormwater quality treatment infrastructure unit charge at the date of payment (refer to Section 3.5 Infrastructure Unit Charges in Planning Scheme Policy DCA - Administration for details of the stormwater quality treatment infrastructure unit charge currently in force).

NOTE 3 SCHEDULE DC 4

Unit Charges

a.	For convenience, the infrastructure unit charge for stormwater quality treatment infrastructure is contained in the Local Government’s Scale of Fees and Charges.

NOTE 4 SCHEDULE DC 4

EXAMPLES

(1)	(a)	It is proposed to reconfigure 3 hectares of land at Coolum Beach on the boundaries of Precincts 4 (Local Centre) and 9 (Neighbourhood Residential) into:

(A): 1 lot (8000m2) for future unspecified shops;

(B): 1 lot (5000m2) for future house sites (unspecified number of lots)

(C): 19 residential lots on 1.7 hectares comprising 15 traditional house lots and 4 courtyard lots.

(b)	No previous stormwater quality treatment infrastructure contributions were paid nor is the land subject to infrastructure credits.

(c)	The stormwater quality treatment infrastructure demand for the proposed development using the rates outlined in Table 2 (a) and Table 2 (b) is as follows:-

A

8000 m2

As this proposed lot is a ‘Management Lot’ the stormwater quality demand factor is equivalent to a single detached house – refer Table 2(a) or Table 2(b)

B

5000 m2

As this proposed lot is a ‘Management Lot’ the stormwater quality demand factor is equivalent to a single detached house – refer Table 2(a) or Table 2(b)

C

1.7 ha

As there is a ‘residential lot’ proposal for the land use both Table 2 (a) and Table 2 (b) to determine the demand factor and choose whichever Table calculates the highest demand factor (i.e. cu)

The demand for a single detached house

= 1 cu      

The demand for a single detached house

= 1 cu      

10 cu/ha x 1.7 ha

= 17 cu       

OR

15 trad. lots x 1 cu

= 15 cu

4 c’yard lots x 1 cu

= 4 cu

= 19 cu       

(d)	The stormwater quality treatment infrastructure demand for the development (A)

= 21 cu

(e)	As the land is not subject to infrastructure credits nor the subject of previous stormwater quality treatment infrastructure contributions the existing demand is that allowed for a single detached house (refer to ‘B’ in the calculation formula).

(f)	The demand for a detached house is 1cu (refer to ‘B’ in the calculation formula)

(B) = 1 cu

(g)	The increase in demand is A – B = 20 cu

(h)	The infrastructure contribution is –

20 x 2,192   	(from Schedule DC4 (6))
43,840 x $1.0762	(Infrastructure Unit Charge)

= $47,180.61

(2)	(a)	It is proposed to extend by 500m2 an existing 1000m2 shopping centre at Kuluin / Kunda Park

(b)	As this is an extension to an existing use only the demand factor rates for defined uses apply.

(c)	The shop is on land (3000m2) within the ‘Local Centre’ Precinct.

(d)	No previous stormwater quality treatment infrastructure contributions were paid nor is the land subject to infrastructure credits.

(e)	The stormwater quality treatment infrastructure demand for the proposed development using the rates outlined in Table 2(b) is as follows –

Use Table 2 (b) (Defined Uses) to determine the demand factor for a ‘shopping complex’.

0.2 cu  x  3000m2

100m2

= 6 cu

(f)	The stormwater quality treatment infrastructure demand for the development

(A) = 6 cu.

(g)	The existing stormwater quality treatment infrastructure demand for the shopping centre is as follows –

2300m2    x     0.2 cu             B= 4.6 cu

100m2

(To ascertain the demand factor for an existing use only the site area, eg buildings, car-parking, landscaping, etc approved or actually used for the development is used - in this example 2300 m2).

(h)	The increase in infrastructure demand is A – B which equals 1.4 cu.

(i)	The infrastructure contribution is –

1.4  x  2,192                   (from Schedule DC4 (6)

3,068.80 x  $1.0762          (Infrastructure Unit Charge)

= $3,302.64

(3)	(a)	In this example assume the same parameters as outlined in example (2) except that 	previous contributions of $ 4000 were paid for the existing centre.

(b)	The stormwater quality treatment infrastructure demand for the development

(A) = 6 cu (refer example 2).

(c)	The existing cu demand is to be equal to the cu on which the previous payment was determined.  It was ascertained that the $4,000 previous payment was determined using 4 cu. The 4 cu becomes the existing use demand factor.

(d)	The increase in infrastructure demand is A-B which equals 2 cu.

(e)	The infrastructure contribution is –

2 x 2,192                       (from Schedule DC 4 (6)

4,384 x $1.0762               (Infrastructure Unit Charge)

= $4,718.06

(4) 	(a)	It is proposed to demolish an existing fabrication industry workshop (2000m2 GFA) to 	construct a 2500m2 shopping centre at Kunda Park.

(b)	As this is a change to an existing use only the demand factor rates for defined uses apply.

(c)	The land is 8000m2 within the ‘Local Centre’ Precinct.

(d)	No previous stormwater quality treatment infrastructure contributions were paid nor is the land subject to infrastructure credits.

(e)	The stormwater quality treatment infrastructure demand for the proposed 	development using the rates outlined in Table 2 (b) is as follows:-

Use Table 2 (b) (Defined Uses) to determine the demand factor for a ‘shopping complex’

0.2cu   x  8000m2

100m2

= 16 cu

(f)		The stormwater quality treatment infrastructure demand for the development

(A) = 16 cu.

(g)	The existing stormwater quality treatment infrastructure demand for the fabrication 	industry (‘General Industry’) is as follows —

4000 m2     x    0.18 cu          	B = 7.2 cu

100 m2

(To ascertain the demand factor for an existing use only the site area eg, buildings, carparking, hard-surfaced storage, etc approved or actually used for the development is used - in this example 4000 m2).

(h)	The increase in infrastructure demand is A – B which equals 8.8 cu.

(i)	The infrastructure contribution is —

8.8  x  2,192                      (from Schedule DC 4 (6)

19,289.60 x  $1.0762        (Infrastructure Unit Charge)

= $20,759.47

(5)	(a)	It is proposed to demolish existing shops (2000m2  GFA) and construct 120 two 	bedroom dwelling units and 1000m2  shops at Maroochydore.

(b)	The land is 8000 m2 within the ‘Multi-Storey Residential’ Precinct.

(c)	No previous stormwater quality treatment infrastructure contributions were paid nor is the land subject to infrastructure credits.

(d)	The stormwater quality treatment infrastructure demand for the proposed 	development using the rates outlined in Table 2 (a) and Table 2 (b) is as follows –

Use both Table 2 (a) and Table 2 (b) to determine the demand factor and choose whichever Table calculates the highest demand factor (i.e. cu).

Using Table 2 (a) the demand is:

71.42 cu  x  8000m2

10000 m2

=  57.136 cu                           

OR

Using Table 2 (b) the demand is:

Residential Demand:

120 units  x  0.67 cu / du    =    80.4 cu 	

Commercial Demand:

8000 m2   x  0.2 cu        =    16 cu	

100 m2

Total Demand  =  96.4 cu                 

(e)	The stormwater quality treatment infrastructure demand for the development (A)

= 96.4cu

(f)	The existing stormwater treatment infrastructure demand for the shops is as 	follows:-

5000 m2   x   0.2 cu                  B = 10 cu

100m2

(To ascertain the demand factor for an existing use only the site area, eg. Buildings, carparking, etc approved or actually used for the development is used – in this example 5000m2).

(g)	The increase in infrastructure contribution is A – B which equals 86.4 cu.

(h)	The infrastructure contribution is –

86.4   X  2,192                	(from Schedule DC 4 (6))

189,388.80  X  $1.0762             (Infrastructure Unit Charge)

=  $203,820.22

Notes:

cu = chargeable unit

du = dwelling unit

GFA = Gross Floor Area

ha = hectare

Stormwater Quality Treatment Demand Factor Rates

(9)	The stormwater quality treatment demand factor rates for the various precinct classes within each Planning Area outlined in Volume 3 of this Planning Scheme are shown in Table 2 (a).

(10)	The stormwater quality treatment demand factor rates for the various uses outlined in section 3.3 (Use Definitions) Volume 1 of this Planning Scheme are shown in Table 2 (b).

(11)	Where calculating the proposed demand requires the use of both Table 2 (a) and Table 2 (b) for determining the stormwater quality treatment demand factor rate, the table that calculates the highest demand factor rate is to be used as the stormwater quality treatment demand factor.

Table 2 (a): Stormwater Quality Demand Factor Rates for General and Specific Precincts

| Precinct | No | Planning Area | Stormwater Quality Demand Factor* |
| --- | --- | --- | --- |
| Business and Industry | All Precincts |  | 0.18cu/100m2 site area |
| Core Industry | All Precincts |  | 0.18cu/100m2 site area |
| General Rural Lands | All Precincts |  | N/A |
| Hillslope Residential | All Precincts |  | 5cu/ha |
| Local Centre | All Precincts |  | 0.2cu/100m2 site area |
| Master Planned Community | 9 | Maroochydore | 34cu/ha (Residential Uses) or 0.2cu/100m2 site area (Commercial or Industrial Uses) |
|  | 10 | Maroochydore | 34cu/ha (Residential Uses) or 0.2cu /100m2 site area (Commercial or Industrial Uses) |
|  | 11 | Maroochydore | 34cu/ha (Residential Uses) or 0.2cu /100m2 site area (Commercial or Industrial Uses) |
|  | 15 | Maroochydore | 28cu/ha (Residential Uses) or 0.2cu /100m2 site area (Commercial or Industrial Uses) |
|  | 28 | Nambour | 11cu/ha (Residential Uses) or 0.2cu /100m2 site area (Commercial or Industrial Uses) |
|  | 4 | Sippy Downs | 12.5cu/ha (Residential Uses) or 0.2cu /100m2 site area (Commercial or Industrial Uses) |
|  | 5 | Sippy Downs | 12.5cu/ha (Residential Uses) or 0.2cu /100m2 site area (Commercial or Industrial Uses) |
|  | 8 | Sippy Downs | 12.5cu/ha (Residential Uses) or 0.2cu /100m2 site area (Commercial or Industrial Uses) |
|  | 11 | Sippy Downs | 12.5cu/ha (Residential Uses) or 0.2cu /100m2 site area (Commercial or Industrial Uses) |
|  | 16 | North Shore | 59cu/ha (Residential Uses) or 0.2cu /100m2 site area (Commercial or Industrial Uses) |
|  | 8 | Mt. Coolum | 31cu/ha (Residential Uses) or 0.2cu /100m2 site area (Commercial or Industrial Uses) |
|  | 9 | Mt. Coolum | 11cu/ha (Residential Uses) or 0.2cu /100m2 site area (Commercial or Industrial Uses) |
|  | All other precincts |  | 20cu/ha (Detached Housing only) or use the precinct or precincts from this table that most closely align with the proposed development |

| Precinct | No | Planning Area | Stormwater Quality Demand Factor* |
| --- | --- | --- | --- |
| Mixed Housing | 13 | Maroochydore | 42cu/ha |
|  | 17 | Maroochydore | 38cu/ha |
|  | 20 | Maroochydore | 38cu/ha |
|  | 23 | Maroochydore | 34cu/ha |
|  | 25 | Maroochydore | 42cu/ha |
|  | 27 | Maroochydore | 38cu/ha |
|  | 3 | Nambour | 25cu/ha |
|  | 4 | Nambour | 25cu/ha |
|  | 7 | Mooloolaba | 42cu/ha |
|  | 8 | Mooloolaba | 42cu/ha |
|  | 13 | Mooloolaba | 44cu/ha |
|  | 2 | Buderim | 27cu/ha |
|  | 5 | Alexandra Headland/ Cotton Tree | 38cu/ha |
|  | 8 | Alexandra Headland/ Cotton Tree | 44cu/ha |
|  | 10 | Alexandra Headland/ Cotton Tree | 38cu/ha |
|  | 11 | Alexandra Headland/ Cotton Tree | 42cu/ha |
|  | 4 | Kuluin/Kunda Park | 22cu/ha |
|  | 13 | North Shore | 22cu/ha |
|  | 2 | Mt. Coolum | 44cu/ha |
|  | 3 | Coolum Beach | 63cu/ha |
|  | All other precincts |  | 35.07cu/ha |
| Multi-storey Residential | All Precincts |  | 71.42cu/ha |
| Neighbourhood Residential | All Precincts |  | 10cu/ha |
| Special Purpose | All Precincts |  | Use the precinct or precincts from this table that most closely align with the proposed development |
| Sustainable Cane Lands | All Precincts |  | N/A |
| Sustainable Horticultural Lands | All Precincts |  | N/A |
| Sustainable Pastoral Lands | All Precincts |  | N/A |
| Sustainable Rural Residential | All Precincts |  | 1.42cu/ha |
| Town Centre Core | All Maroochydore Precincts |  | 200cu/ha (Residential Uses) or 0.2cu/100m2 site area (Commercial or Industrial Uses) |
|  | Nambour |  | 71.42cu/ha (Residential Uses) or 0.2cu /100m2 site area (Commercial or Industrial Uses) |
|  | Sippy Downs |  | 71.42cu/ha (Residential Uses) or 0.2cu /100m2 site area (Commercial or Industrial Uses) |
|  | Mooloolaba |  | 200cu/ha (Residential Uses) or 0.2cu /100m2 site area (Commercial or Industrial Uses) |
| Town Centre Frame | All Maroochydore and Mooloolaba Precincts |  | 35.07cu/ha (Residential Uses) or 0.2cu /100m2 site area (Commercial or Industrial Uses) |
|  | Nambour |  | 35.07cu/ha (Residential Uses) or 0.2cu /100m2 site area (Commercial or Industrial Uses) |

| Precinct | Planning Area | Stormwater Quality Demand Factor* |
| --- | --- | --- |
|  | Sippy Downs | 35.07cu/ha (Residential Uses) or 0.2cu /100m2 site area (Commercial or Industrial Uses) |
| Village Centre | All Precincts | 35.07cu/ha (Residential Uses) or 0.2cu /100m2 site area (Commercial or Industrial Uses) |
| Water Resource Catchment Area | All Precincts | N/A |

* For a ‘Management Lot’ the Stormwater Quality Demand Factor is 1.00cu/Lot.

Notes:

ha = hectare

cu = chargeable unit

Table 2(B): Stormwater Quality Treatment Demand Factor Rates for Defined Uses

| Defined Uses and Use Classes | Assessment Unit | Stormwater cu/Unit* |
| --- | --- | --- |
| RESIDENTIAL USES |  |  |
| Accommodation Building | Bed | 0.52 |
| Bed and Breakfast | Lettable Room | 0.52 |
| Caravan Park | Caravan Site | 0.55 |
|  | Relocatable home site | 0.55 |
| Caretakers Residence | 1 bedroom | 0.58 |
|  | 2 bedroom | 0.75 |
|  | 3 or more bedrooms | 0.92 |
| Detached House | Lot | 1.00 |
| Display Home | As per detached house or dual occupancy or multiple dwelling units, depending on nature of development |  |
| Dual Occupancy | 1 bedroom | 0.58 |
|  | 2 bedroom | 0.75 |
|  | 3 or more bedrooms | 0.92 |
| Home-Based Business | Lot | Exempt provided equivalent demand for a detached house is not exceeded, otherwise to be assessed on use. |
| Institutional Residence | Bed | 0.52 |
| Integrated Tourist Facility | Bed | 0.52 |
|  | For permanent residential accommodation as per detached house or dual occupancy or multiple dwelling units, depending on nature of development. |  |
| Motel (includes hotel accommodation) | Per unit | 0.52 |
| Multiple Dwelling Units | 1 bedroom | 0.52 |
|  | 2 bedroom | 0.67 |
|  | 3 or more bedrooms | 0.86 |
| Residential Care facility | Per self contained dwg | 0.52 |
|  | Per hostel unit | 0.52 |
|  | Per nursing care bed | 0.52 |
| Retirement Village | 1 bedroom | 0.52 |
|  | 2 bedroom | 0.67 |
|  | 3 or more bedrooms | 0.86 |

| Defined Uses and Use Classes |  |  |  | Assessment Unit |  |  | Stormwater cu/Unit* |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RURAL USES |  |  |  |  |  |  |  |  |
| Agriculture, Animal Keeping, Animal Husbandry, Aquaculture, Forestry, Intensive Animal Husbandry, Roadside stall, Stables |  |  |  |  |  |  | Exempt |  |
| Rural Service Industry |  |  |  |  |  |  | Exempt |  |
| Winery |  |  |  |  |  |  | Exempt |  |
| COMMERCIAL USES |  |  |  |  |  |  |  |  |
| Adult Product Shop |  |  |  | per 100m2 site area |  |  | 0.20 |  |
| Art & Craft Centre |  |  |  | per 100m2 site area |  |  | 0.20 |  |
| Convenience Restaurant |  |  |  | per 100m2 site area |  |  | 0.20 |  |
| Fast Food Store |  |  |  | per 100m2 site area |  |  | 0.20 |  |
| Funeral Parlour |  |  |  | per 100m2 site area |  |  | 0.20 |  |
| Garden Centre |  |  |  | per 100m2 site area |  |  | 0.20 |  |
| Hotel |  |  |  | per 100m2 site area |  |  | 0.20 |  |
| Market |  |  |  | As determined by Council |  |  |  |  |
| Medical Centre |  |  |  | per 100m2 site area |  |  | 0.20 |  |
| Office |  |  |  | per 100m2 site area |  |  | 0.20 |  |
| Restaurant |  |  |  | per 100m2 site area |  |  | 0.20 |  |
| Shop (including General Store) |  |  |  | per 100m2 site area |  |  | 0.20 |  |
| Shopping Complex |  |  |  | per 100m2 site area |  |  | 0.20 |  |
| Showroom |  |  |  | per 100m2 site area |  |  | 0.20 |  |
| Veterinary Clinic |  |  |  | per 100m2 site area |  |  | 0.20 |  |
| INDUSTRIAL USES |  |  |  |  |  |  |  |  |
| Car Washing Station |  |  |  | per 100m2 site area |  |  | 0.18 |  |
| Environmentally Assessable Industry |  |  |  | per 100m2 site area |  |  | 0.18 |  |
| Extractive Industry |  |  |  |  |  |  | Exempt |  |
| General Industry |  |  |  | per 100m2 site area |  |  | 0.18 |  |
| Landscape Supplies |  |  |  | As determined by Council |  |  |  |  |
| Light Industry |  | Laundromat |  | per 100m2 site area |  |  | 0.18 |  |
|  |  | Hot bread kitchen/retail bakery |  | per 100m2 site area |  |  | 0.18 |  |
|  |  | All other uses |  | per 100m2 site area |  |  | 0.18 |  |
| Sales or Hire Yard |  |  |  | As determined by Council |  |  |  |  |
| Service Station |  |  |  | per 100m2 site area |  |  | 0.18 |  |
| Storage Yard |  |  |  | As determined by Council |  |  |  |  |
| Transport Station |  |  |  | As determined by Council |  |  |  |  |
| Vehicle Depot |  |  |  | As determined by Council |  |  |  |  |
| Vehicle Repair workshop |  |  |  | per 100m2 site area |  |  | 0.18 |  |
| Warehouse |  |  |  | per 100m2 site area |  |  | 0.18 |  |
| OTHER USES |  |  |  |  |  |  |  |  |
| COMMUNITY USE |  | Child Care Centre |  | per 100m2 site area |  |  | 0.20(1) |  |
|  |  | Local Utility |  | per 100m2 site area |  |  | 0.20(1) |  |
|  |  | Major Utility |  | per 100m2 site area |  |  | 0.20(1) |  |
|  |  | Telecommunications Facility |  | per 100m2 site area |  |  | 0.20(1) |  |
| SPECIAL USE |  | Cemetery |  |  |  |  | As determined by Council |  |
|  |  | Church |  | per 100m2 site area |  |  | 0.20(1) |  |
|  |  | Community Meeting Hall |  | per 100m2 site area |  |  | 0.20(1) |  |
|  |  | Crematorium |  | per 100m2 site area |  |  | 0.20(1) |  |
|  |  | Educational Establishment |  | per 100m2 site area |  |  | 0.20(1) |  |
|  |  | Emergency Services |  |  |  |  | As determined by Council |  |
|  |  | Hospital |  | per 100m2 site area |  |  | 0.20(1) |  |
| Defined Uses and Use Classes |  |  | Assessment Unit |  |  |  | Stormwater cu/Unit* |  |
| RECREAT– IONAL USES | INDOOR RECREATION |  | Amusement Centres |  | per 100m2 site area |  | 0.20 |  |
|  |  |  | Gyms |  | per 100m2 site area |  | 0.20 |  |
|  |  |  | Indoor Sports Centre |  | per 100m2 site area |  | 0.20 |  |
|  |  |  | Licensed Club |  | per 100m2 site area |  | 0.20 |  |
|  |  |  | Unlicensed Club |  | per 100m2 site area |  | 0.20 |  |
|  |  |  | Night Club |  | per 100m2 site area |  | 0.20 |  |
|  |  |  | Theatre / Cinema |  | per 100m2 site area |  | 0.20 |  |
|  |  |  | All other uses |  | As determined by Council |  |  |  |
|  | OUTDOOR RECREATION |  |  |  |  |  | As determined by Council |  |
| OTHER USES | CAR PARK |  |  |  | per 100m2 site area |  | 0.20 |  |

(1) For uses located in the Business and Industry, Core Industry or Hillslope Residential Precincts 		the equivalent demand is 0.18cu/100m2 site area

* For a ‘Management Lot’ the Stormwater cu/Unit is 1.00cu/Lot.

### APPENDIX 1

### FIGURE 4.2.1: STORMWATER QUALITY IMPROVEMENT INFRASTRUCTURE – TREATMENT STYLES

### APPENDIX 2

### DESIRED STANDARDS OF SERVICE FOR STORMWATER QUALITY

A2.1 	The desired standard of service (DSS), or performance objectives, for which stormwater quality is planned/designed to, is determined by the community through the identification of environmental values.

A2.2	Environmental values describe the expectations or aspirations a community have for their waterways.

A2.3	These values represent more than just the community’s use of the waterway but also include their visions for the waterway and their perceptions of its potential.

NOTE on A2.1 – A2.3

- It is acknowledged that in some cases, due to local circumstances, the desired standard of service may not be met. In these situations, Stormwater Quality Treatment infrastructure aims to meet the standards to the greatest degree practicable. Knowing how the community value a waterway is a fundamental step in developing a responsible stormwater management plan for the catchment. For instance, strategies for the management of stormwater for a catchment whose receiving waters have been valued as secondary recreation should differ from those whose receiving waters have been valued as drinking water.

A2.4 	Environmental values are agreed to through consultation with the community.

A2.5	The values can only be decided by people, there are no algorithms or formulas that can be used to mimic the community’s wishes.

A2.6	The environmental values adopted to determine these Infrastructure Contributions were determined during the South East Queensland Regional Water Quality Management Strategy, September 2001.

A2.7	It is possible to determine environmental values on a local, regional or Shire wide level.

NOTE on A2.7 ENVIRONMENTAL VALUES

- The environmental values adopted in the Draft Urban Stormwater Quality Management Plan (USQMP) for Maroochy Shire November 2002 are regional values and are appropriate for the purposes of developing a Shire wide strategy to achieve Water Quality Objectives (WQOs). Local environmental values are assigned to receiving waters where a local strategy can be applied and monitored. Shire wide environmental values refer to the collective regional environmental values derived for each catchment in the USQMP.

A2.8 	Following the determination of the community’s nominated environmental values, a measurable objective is required to monitor the compliance or attainment of such values. These objectives are known as the Water Quality Objectives.

NOTE on A2.8 WATER QUALITY OBJECTIVES

- Water Quality Objectives are set using either state or national guidelines – supplemented with local studies (if available). Water Quality Objectives are defined in EPP Water as being numerical concentration levels or statements for indicators that protect a stated environmental value.

- Water quality models are used as a tool to evaluate the potential impacts of development on water quality. The Draft Urban Stormwater Quality Management Plan for Maroochy Shire (USQMP), November 2002 developed water quality models for 15 of the Shire’s creek catchments using the Model for Urban Stormwater Improvement Conceptualisation (MUSIC) developed by the Cooperative Research Centre for Catchment Hydrology (CRC-CH).

- Pollutant concentrations of TSS, TN and TP were calculated for an ultimate development scenario of each catchment and compared against the WQO’s assigned to each catchment

A2.9 	The Maroochy Plan (2000) and associated planning scheme policies require developers to be responsible for the stormwater quality within and discharging from their sites.

A2.10 	Planning Scheme Policy 11 Flooding and Stormwater Management Report Requirements states that the environmental values and associated water quality objectives may be determined by a Catchment Management Plan or in the interim adopt values supplied by Council.

A2.11	Assuming these criteria are complied with for all new development, leads to the assumption that the cost of achieving localised environmental values via the adoption of Water Sensitive Urban Design (WSUD) will be borne by future development.

NOTE on A2.9 – A2.11 STORMWATER QUALITY TREATMENT

- Stormwater quality can be improved by applying the principles of Water Sensitive Urban Design (WSUD).  WSUD principles encompass all aspects of water cycle management including water supply, sewerage and stormwater management. Structural measures such as stormwater quality improvement devices SQIDs (i.e. swales, bio-retention, GPTs and wetlands) are one of the elements of WSUD.

- The Draft Urban Stormwater Quality Management Plan for Maroochy Shire (USQMP) used the water quality model developed for each catchment to determine the number and size of a range of SQIDs that are required to meet the WQO’s and hence deliver the desired environmental values to the community.

- Generic development layouts and treatment efficiencies were adopted for the water quality models and hence numbers and sizes of devices are indicative only of Best Management Practices (BMPs). On this basis it is not the intention of the USQMP to develop a program of SQIDs to be constructed in each catchment.

The onus is upon the applicant to provide evidence of any previous infrastructure contributions paid to Council

Refer to Division 10 – Glossary of Terms in Planning Scheme Policy DCA – Administration for an explanation of the term “existing use demand entitlement”.